BLK16E-0291-38X38-B V1_01 Interface Description

P1 Power Ethernet

This interface is not welded

P3 IRcut

P3 Infrared

P6 Alarm Audio

| sign | Socket | Specific No | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P1 | J3 | 1 | LED/PHY_AD0 | LED |
|  |  | 2 | LED/PHY_AD1 | LED |
|  |  | 3 | ETHTX+ | Ethernet signal |
|  |  | 4 | ETHTX- | Ethernet signal |
|  |  | 5 | ETHRX+ | Ethernet signal |
|  |  | 6 | ETHRX- | Ethernet signal |
|  |  | 7 | GND | GND |
|  |  | 8 | 12V | 12V DC input |
| P3 | J4 | 1 | Infrared input | Infrared input |
|  |  | 2 | GND | GND |
|  |  | 3 | +3.3V | 3.3V DC input |
| P3 | J5 | 1 | BOUT | IRCUT output |
|  |  | 2 | AOUT | IRCUT output |
| P5 | J2 | 1 | UART_RX |  |
|  |  | 2 | UART_TX |  |
|  |  | 3 | GND | GND |
| P6 | J3 | 1 | AC_OUT | AC_OUT |
|  |  | 2 | AC_IN | MIC_P |
|  |  | 3 | GND | GND |
|  |  | 4 | ALARM_GPIO | ALARM_IN |
|  |  | 5 | GND | GND |