P1 Ethernet

P6 audio/video

P3 IRcut

P2 Infrared interface

P4 power

| sign | Socket | Specific No | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P1 | J3 | 1 | ETHRX- | Ethernet signal |
|  |  | 2 | ETHRX+ | Ethernet signal |
|  |  | 3 | LED/PHY_AD3 | LED |
|  |  | 4 | ETHTX- | Ethernet signal |
|  |  | 5 | ETHTX+ | Ethernet signal |
|  |  | 6 | LED/PHY_AD0 | LED |
| P2 | J2 | 1 | Infrared input | Infrared input |
|  |  | 2 | GND | GND |
|  |  | 3 | +5V | 5V DC input |
| P3 | J4 | 1 | BOUT | IRCUT output |
|  |  | 2 | AOUT | IRCUT output |
| P4 | J7 | 1 | NC | Undefined |
|  |  | 2 | NC | Undefined |
|  |  | 3 | GND | GND |
|  |  | 4 | 12V | 12V DC input |
| P5 | J1 | 1 | UART_RX |  |
|  |  | 2 | UART_TX |  |
|  |  | 3 | GND | GND |
| P6 | J6 | 1 | AC_OUT | AC_OUT |
|  |  | 2 | MIC_P | MIC_P |
|  |  | 3 | GND | GND |
|  |  | 4 | ALARM_IN | ALARM_IN |
|  |  | 5 | GND | GND |